# Unit 1.4 Wired and Wireless Networks – Lesson 1

## Activity 1 – Local area network (LAN) vs Wide area network (WAN) (Answers)

### Higher ability

Research the key areas below about each type of network. Complete the boxes for each network.

| Key point to research | LAN | WAN |
| --- | --- | --- |
| Area it covers | Small geographical area that covers a single building or a single site | Large geographical area e.g. several sites, town, county, country, continent |
| Connection | Wireless Ethernet Cable (Wired) Fibreoptic Use of Network Interface Cards Use of Wireless Adapter Connected via a Router / Switch | Phone lines Internet connection Fibre Optic Satellite Public network systems Leased lines Dedicated lines |
| Speed | Very High Speed (up to 1000mbps) | Slower speed (aprox 150 mbps) |
| Cost | Cheaper to set up – need for additional hardware such as a router and cables (although could use wireless) | Using a dedicated line would be costly for a company. Using a leased line would incur monthly subscription costs WAN would require ongoing costs of using external infrastructure |
| Security | More control over security Anti-virus Firewalls User names / Passwords Account management | Less control over security especially hackers & DDOS attack. Would need to configure firewalls & up-to-date anti-virus |
| Example | A school network A home network | A council network Argos network |

Extension:

- You have recently had an Internet connection put into your home and decide to set up a LAN so that you can connect your PC, tablet, mobile phone and game consoles to share The Internet and also photos/videos from these devices.

- Why is a LAN the most appropriate choice?

Because it is a single site – over a small geographical area

- What hardware would you need to buy?

Would need a connection device like a router, potentially Ethernet cables to link the PC and the consoles for the best speeds. Would also need a Network Interface Card for the PC (or a wireless adapter)

- How would each device connect to the network?

Either by Ethernet wired cable or through wireless (wifi)

- Discuss the factors that would affect the speed of the network

The type of Ethernet cable would affect the speed – Cat 5e / Cat6 cable would offer the fastest speeds. If devices are connecting wirelessly – walls and other interference from other radio waves could affect the speed of the network e.g. how far from the router is the device.

- If a company wanted to connect to a WAN – they may make use of a Dedicated Line or a Leased Line – discuss the difference between these two.

Leased line – a company would pay a monthly subscription cost to rent a line (which is external infrastructure) that would allow devices to connect over a large geographical area. The company that the line is being rented from would have the responsibility of maintenance. Dedicated line would be one in which the company would pay to have an external cable dedicated to their data transmission. If there were any problems with the cable then they would be responsible for maintenance. This can be very costly but offers great transfer speeds.

## Activity 1 – Local area network (LAN) vs Wide area network (WAN)

### Low ability

Research the key areas below about each type of network. Complete the boxes for each network.

| Key point to research | LAN | WAN |
| --- | --- | --- |
| How big is the area that this type of network covers? - Geographically small? - Geographically large? |  |  |
| How are computers normally connected together in this type of network - Wired (Ethernet) - Wirelessly - Dedicated Lines - Leased Line - The Internet - Satellite - Phone Lines |  |  |
| What is the average speed of data transmission on this type of network? - Kbps? - Mbps? - Gbps? - Tbps? |  |  |
| What kind of costs are involved in setting up this type of network? - Initial purchase of hardware? - On-going costs (subscriptions)? - Cable costs |  |  |
| How secure is this type of network? - What security risks are there? - What could be done to help the security? |  |  |
| Can you give an example of this type of network? |  |  |

We’d like to know your view on the resources we produce.  By clicking on ‘Like’ or ‘Dislike’ you can help us to ensure that our resources work for you.  When the email template pops up please add additional comments if you wish and then just click ‘Send’.  Thank you.

If you do not currently offer this OCR qualification but would like to do so, please complete the Expression of Interest Form which can be found here: www.ocr.org.uk/expression-of-interest

OCR Resources: the small print
OCR’s resources are provided to support the teaching of OCR specifications, but in no way constitute an endorsed teaching method that is required by the Board, and the decision to use them lies with the individual teacher.   Whilst every effort is made to ensure the accuracy of the content, OCR cannot be held responsible for any errors or omissions within these resources. 
© OCR 2016 - This resource may be freely copied and distributed, as long as the OCR logo and this message remain intact and OCR is acknowledged as the originator of this work.

OCR acknowledges the use of the following content: n/a

Please get in touch if you want to discuss the accessibility of resources we offer to support delivery of our qualifications: resources.feedback@ocr.org.uk

OCR Resources: the small print
OCR’s resources are provided to support the delivery of OCR qualifications, but in no way constitute an endorsed teaching method that is required by the Board, and the decision to use them lies with the individual teacher.   Whilst every effort is made to ensure the accuracy of the content, OCR cannot be held responsible for any errors or omissions within these resources.
© OCR 2016 - This resource may be freely copied and distributed, as long as the OCR logo and this message remain intact and OCR is acknowledged as the originator of this work.

OCR acknowledges the use of the following content: n/a

Please get in touch if you want to discuss the accessibility of resources we offer to support delivery of our qualifications: resources.feedback@ocr.org.uk

We’d like to know your view on the resources we produce.  By clicking on ‘Like’ or ‘Dislike’ you can help us to ensure that our resources work for you.  When the email template pops up please add additional comments if you wish and then just click ‘Send’.  Thank you.

Whether you already offer OCR qualifications, are new to OCR, or are considering switching from your current provider/awarding organisation, you can request more information by completing the Expression of Interest form which can be found here: www.ocr.org.uk/expression-of-interest

Looking for a resource? There is now a quick and easy search tool to help find free resources for your qualification: 
www.ocr.org.uk/i-want-to/find-resources/